How to configure and use the PSSI on the STM32U5

Introduction :

The PSSI stands for Parallel synchronous slave interface and was introduced in our high performance STM32U5 family. The PSSI can be used as a fast interface between two STM32 microcontrollers or between an STM32 and an MPU or FPGA. This article will show how to configure and use this interface on a STM32U575.

Prerequisites :

- Hardware :

  - Micro USB cable used to power the Nucleo board from a host machine and to load the code into the STM32

  - 2x Nucleo-U575ZI-Q

  - connection cables

- Software :

  - STM32CubeMX

  - STM32CubeIDE

Steps :

- Open STM32CubeMX and select  “ ACCESS TO BOARD SELECTOR“

- Select the correct board that you are working on. For this example, we are working with the “NUCLEO-U575ZI-Q”

Start the project without initializing any peripherals.

- Go to multimedia and select “PSSI” and configure it to the following :

- enable the NVIC Interrupt of the PSSI from the NVIC Settings .

- Go to System Core and configure two GPDMA channels one for transmission and the second for recieving as follows(In this exemple we went with channel 12 and 13 note that if 2D addressing are available they must be used as refernced in the application note):

- Enable the ICACHE for a performance boost :

- Change the time base source to SysTick to boost the performance of the AHB2 :

- Enable the USART1 to display the transferred messages using the serial link via ST-Link with the following configuration:

- To ensure the transfer and receiving of data using PSSI you need to create two different linked lists with the following configurations each linked list will be linked to a channel of the GPDMA:

- Edit the clock configuration to the following to obtain the maximum performance out of the GPDMA and the PSSI :

- Clear all the unused pins and generate the project we used for this use case the STM32CubeIDE as a preferred toolchain  :

Code :

1. In file “Core/Inc/main.h” add the following lines in the Private includes section:

/* USER CODE BEGIN Private defines */

/* Definition for TIMx clock resources */

#define TIMx                           TIM1

#define TIMx_CLK_ENABLE()              __HAL_RCC_TIM1_CLK_ENABLE()

/* USER CODE END Private defines */

2. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN Includes ” section:

/* USER CODE BEGIN Includes */

#include "stdio.h"

#include <stdlib.h>

#include "string.h"

/* USER CODE END Includes */

3. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN PTD” section(note that commenting or uncommenting the variable MASTER_BOARD in this section determine if this is the code of the master or the slave):

/* USER CODE BEGIN PTD */

TIM_HandleTypeDef    htim;

TIM_OC_InitTypeDef   sConfig; /* Timer Output Compare Configuration Structure declaration */

//#define MASTER_BOARD

#define  TIMx                TIM1

#define  PERIOD_VALUE       (uint32_t)(1000 - 1)              /* Period Value             */

#define  PULSE1_VALUE       (uint32_t)(PERIOD_VALUE/2)        /* Capture Compare 1 Value  */

/* USER CODE END PTD */

4. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN PM” section:

/* USER CODE BEGIN PM */

uint32_t uhPrescalerValue = 0;

uint32_t Request_received = 0 ;

uint32_t data_cmp = 0 ;

uint32_t i;

uint32_t PSSI_HAL_PSSI_TransmitComplete_count = 0;

uint32_t PSSI_HAL_PSSI_ReceiveComplete_count = 0;

uint32_t PSSI_HAL_PSSI_ErrorCallback_count = 0;

#ifndef MASTER_BOARD

//ALIGN_32BYTES (char     pData8_S_TRSMT[64] ="Hello from Slave");  /* Data to transmit from Slave */

#define ALIGN_32BYTES __attribute__((aligned(32)))

char pData8_S_TRSMT[64] ALIGN_32BYTES = "Hello from Slave";

ALIGN_32BYTES char pData8_S_RCV[64];

#else

ALIGN_32BYTES (char     pData8_M_RCV[64]);

ALIGN_32BYTES (char     pData8_M_TRSMT[64]); /* Data to transmit from Master */

#endif

/* USER CODE END PM */

5. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN PV” section:

extern DMA_QListTypeDef  PSSITXQueue;

extern DMA_QListTypeDef  PSSIRXQueue;

6. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN PFP” section:

/* USER CODE BEGIN PFP */

#ifdef MASTER_BOARD

static void Timer_Config(TIM_HandleTypeDef *htim);

static void CLK_On(void);

static void CLK_Off(void);

static uint32_t Fetch_Slave_Request(char Received_Buffer[64]);

#endif /*MASTER_BOARD*/

#define PUTCHAR_PROTOTYPE int __io_putchar(int ch)

/* USER CODE END PFP */

7. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN 0” section:

/* USER CODE BEGIN 0 */

int _write(int file, char *ptr, int len) {

HAL_UART_Transmit(&huart1, (uint8_t*) ptr, len, HAL_MAX_DELAY);

return len;

}

/* USER CODE END 0 */

8. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN 2” section:

/* USER CODE BEGIN 2 */

HAL_DMAEx_List_LinkQ(&handle_GPDMA1_Channel13, &PSSITXQueue);

__HAL_LINKDMA(&hpssi, hdmatx, handle_GPDMA1_Channel13);

HAL_DMAEx_List_LinkQ(&handle_GPDMA1_Channel12, &PSSIRXQueue);

__HAL_LINKDMA(&hpssi, hdmarx, handle_GPDMA1_Channel12);

#ifdef MASTER_BOARD

printf("Hello this is the master board \n\r");

HAL_Delay(500);

/* Configure Tim to generate the clock for PSSI slave */

Timer_Config(&htim) ;

while(1)

{

CLK_On();

while(Request_received != 1)

{

if(HAL_PSSI_Receive_DMA(&hpssi, (uint32_t*)pData8_M_RCV , sizeof(pData8_M_RCV)/4)!= HAL_OK)

{

Error_Handler();

}

while(PSSI_HAL_PSSI_ReceiveComplete_count !=1)

{

/* wait until receiving data is complete */

}

if(Fetch_Slave_Request(pData8_M_RCV) == 1)

{

Request_received = 1;

}

PSSI_HAL_PSSI_ReceiveComplete_count = 0 ;

}

/* Start transmitting the received & modified data */

if(HAL_PSSI_Transmit_DMA(&hpssi, (uint32_t*)pData8_M_TRSMT, sizeof(pData8_M_TRSMT)/4)!= HAL_OK)

{

Error_Handler();

}

HAL_Delay(2);

while(PSSI_HAL_PSSI_TransmitComplete_count != 1)

{

/* wait until transmit data is complete */

}

printf("Transmission completed\n\r");

HAL_Delay(200);

HAL_Delay(20);

/* Transmit done */

Request_received = 0 ;

PSSI_HAL_PSSI_TransmitComplete_count = 0 ;

CLK_Off();

/*Wait 1s before the next loop*/

HAL_Delay(1000);

}

#else

/*PSSI slave Part*/

printf("Hello this is the slave board \n\r");

HAL_Delay(500);

/* Start transmitting the data */

if(HAL_PSSI_Transmit_DMA(&hpssi, (uint32_t*)(pData8_S_TRSMT), sizeof(pData8_S_TRSMT))!= HAL_OK)

{

Error_Handler();

}

while(PSSI_HAL_PSSI_TransmitComplete_count != 1)

{

/* wait until transmit data is complete */

}

HAL_Delay(2);

/* Start receiving the data */

if(HAL_PSSI_Receive_DMA(&hpssi, (uint32_t*)(pData8_S_RCV), sizeof(pData8_S_RCV)/4)!= HAL_OK)

{

Error_Handler();

}

while(PSSI_HAL_PSSI_ReceiveComplete_count != 1)

{

/* wait until receiving data is complete */

}

/*********************************************************************************************/

/* Check first that the transmitted data is received */

for(i = 0; i < strlen(pData8_S_TRSMT) ;i++)

{

if(pData8_S_TRSMT[i] != pData8_S_RCV[i])

{

data_cmp++;

break;

}

}

/*if OK , check then, that the Master has modified the transmitted data*/

if((data_cmp == 0) && (strstr(pData8_S_RCV," Master") != 0))

{ /* Infinite loop */

while (1)

{

printf("data has been modified\n\r");

printf("%s\n\r",pData8_S_RCV);

HAL_Delay(1000);

}

}

else /*KO*/

{

Error_Handler();

}

#endif /*MASTER_BOARD*/

/* USER CODE END 2 */

8. In file “Core/Src/main.c” add the following lines in the “USER CODE BEGIN 4” section:

#ifdef MASTER_BOARD

void Timer_Config(TIM_HandleTypeDef *htim)

{

/* Compute the prescaler value to have TIM1 counter clock equal to 20000 Hz */

uhPrescalerValue = (uint32_t)(250000000 / (20000000)) - 1;

htim->Instance = TIMx;

htim->Init.Prescaler         = uhPrescalerValue;

htim->Init.Period            = PERIOD_VALUE;

htim->Init.ClockDivision     = 0;

htim->Init.CounterMode       = TIM_COUNTERMODE_UP;

htim->Init.RepetitionCounter = 0;

if (HAL_TIM_PWM_Init(htim) != HAL_OK)

{

/* Initialization Error */

Error_Handler();

}

/*##-2- Configure the PWM channel #########################################*/

/* configuration for the channel */

sConfig.OCMode       = TIM_OCMODE_PWM1;

sConfig.OCPolarity   = TIM_OCPOLARITY_HIGH;

sConfig.OCFastMode   = TIM_OCFAST_DISABLE;

sConfig.OCNPolarity  = TIM_OCNPOLARITY_HIGH;

sConfig.OCNIdleState = TIM_OCNIDLESTATE_RESET;

sConfig.OCIdleState  = TIM_OCIDLESTATE_RESET;

/* Set the pulse value for channel 1 */

sConfig.Pulse = PULSE1_VALUE;

if (HAL_TIM_PWM_ConfigChannel(htim, &sConfig, TIM_CHANNEL_1) != HAL_OK)

{

/* Configuration Error */

Error_Handler();

}

}

static void CLK_On(void)

{

/*Start PWM signals generation #######################################*/

/* Start channel 1 */

if (HAL_TIM_PWM_Start(&htim, TIM_CHANNEL_1) != HAL_OK)

{

/* PWM Generation Error */

Error_Handler();

}

}

static void CLK_Off(void)

{

if (HAL_TIM_PWM_Stop(&htim, TIM_CHANNEL_1) != HAL_OK)

{

/* PWM Generation Error */

Error_Handler();

}

}

uint32_t Fetch_Slave_Request(char Received_Buffer[64])

{

uint32_t j;

char REPLY[23] = ": received, from Master" ;

if(strstr(Received_Buffer,"Slave") != 0)

{

for(i=0 ; i < strlen(Received_Buffer) ; i++)

{

pData8_M_TRSMT[i] = Received_Buffer[i];

}

for(j=0;j<strlen(REPLY);j++)

{

pData8_M_TRSMT[i] = REPLY[j];

i++;

}

return 1;

}

else

{

return 0;

}

}

#endif /* MASTER_BOARD */

void HAL_PSSI_TxCpltCallback(PSSI_HandleTypeDef *hpssi)

{

PSSI_HAL_PSSI_TransmitComplete_count++;

}

/**

* @brief PSSI receive complete callback

* @par hppsi: PSSI HAL handle

* @retval None

*/

void HAL_PSSI_RxCpltCallback(PSSI_HandleTypeDef *hpssi)

{

PSSI_HAL_PSSI_ReceiveComplete_count++;

}

/**

* @brief PSSI transfer error callback

* @par hppsi: PSSI HAL handle

* @retval None

*/

void HAL_PSSI_ErrorCallback(PSSI_HandleTypeDef *hpssi)

{

PSSI_HAL_PSSI_ErrorCallback_count++;

}





9. In file “Core/Inc/stm32u5xx_hal_conf.h” uncomment the following line :

#define HAL_TIM_MODULE_ENABLED

10. Enjoy the demo.